AIRFRAME 02

ELECTRIC LANDING GEAR – LAB 3

Name ________________________________________	Bin No. _______________________

Due Date ______________________

A.  CESSNA 310 RETRACTION SYSTEM

Use the Cessna 310 Service Manual and the attached wiring diagram as needed, to answer parts 1 and 2 below.

1.  During the retraction test on the 310, inspect the following:

| a) | The gear retracts and extends easily and without binding or strain. | _____________________________________ |
| --- | --- | --- |
| b) | The warning horn sounds if gear is retracted and throttle is retarded. | _____________________________________ |
| c) | Down and up locks hold gear firmly in position. | _____________________________________ |
| d) | Warning lights show safe operation of gear. | _____________________________________ |
| e) | Ground safety-lock system assures aircraft cannot be retracted on the ground. | _____________________________________ |
| f) | Emergency system extends gear satisfactorily. | _____________________________________ |
| g) | Using the attached wiring diagrams of a Cessna 310 landing gear, color all wires Red that are part of the retraction of the landing gear. | _____________________________________ |
| h) | Using the attached wiring diagrams of a Cessna 310 landing gear, color all wires Blue that are part of the extension of the landing gear. | _____________________________________ |

2.  Troubleshooting

| a) | Where are the up and down limit switches physically located? | _____________________________________ |
| --- | --- | --- |
| b) | What is the first adjustment that should be checked in this system? | _____________________________________ |
| c) | Where is the motor load resistor physically located? | _____________________________________ |
| d) | What would be the indication if wire G25 failed open? | _____________________________________ |

2.  Troubleshooting (Cont)

| e) | What would be the indication if wire G13A failed open? | _____________________________________ |
| --- | --- | --- |
| f) | What would be the indication if wire L61A failed open? | _____________________________________ |
| g) | What would be the indication if the LH Gear Down Switch failed stuck in the N.O. position? | _____________________________________ |
| h) | When should an aircraft be jacked up and the gear operated for inspection? | _____________________________________ _____________________________________ |
| i) | What are 3 subsystems of the total landing gear system that should be inspected for proper operation during the above inspection? | _____________________________________ _____________________________________ |
|  |  | _____________________________________ |
| j) | How does the squat switch prevent the gear from retracting? | _____________________________________ |
| k) | Which landing gear is the squat switch mounted on? | _____________________________________ |
| l) | What are the operating limitations of the electric gear motor? | _____________________________________ |
| m) | What is the full nomenclature of the electric motor? | _____________________________________ |
| n) | Besides the squat switch, what normal operating procedure helps prevent placing the selector in the up position? | _____________________________________ |

# B.  LANDING GEAR INDICATING SYSTEMS

1.  QUESTIONS (use the attached schematic to answer the following questions):

| a. | Position indicators will show the pilot if the landing gear is in what position? |  | _____________________________ |
| --- | --- | --- | --- |
| b. | What color light is used to indicated the gear is down and locked? |  | _____________________________ |
| c. | What signals are used to reduce the possibility of the landing with the gear retracted? |  | _____________________________ |
|  |  |  | _____________________________ |
| d. | What color lights are used to indicate the gear is up and locked? |  | _____________________________ |
| e. | Are the throttle switches open or closed when the warning horn sounds? |  | _____________________________ |
| f. | If all green lights are illuminated and the throttle switches are closed, with the horn sound? | ______________________________ |  |
| g. | What position of the landing gear will cause the amber light to illuminate? | ______________________________ |  |
| h. | If only two green lights are illuminated, what indications will the pilot receive when both throttles are retarded? | ______________________________ |  |
|  |  | ______________________________ |  |
| i. | In what position is the gear per the switch positions? | ______________________________ |  |
| j. | What makes the red light flash on and off? | ______________________________ |  |